	[image: image1.jpg]&
Pé
e OF €0

X
)

)
Yy, 4 2
7 AV
90 Jynony™




	TRDI - WORKHORSE

Checklist 
	National Current Observation Program



	S/N:
	Freq:
	Firmware v:

	Project:
	Station:
	Case S/N (if different):


· A NEW CHECKLIST IS REQUIRED FOR EACH DEPLOYMENT
· USE GMT (Casablanca) FOR COMPUTER TIME
· INITIAL EACH BOX IF SECTION IS COMPLETED BY MORE THAN ONE PERSON
	Lab/Initial Preparations

	INIT
	Task
	Notes

	
	Battery Installed
	Voltage:                                                    VDC

	
	Data Card #0 Write Verified
	If used, backup data if needed and RE ErAsE
	#0 MB:

	
	Data Card #0 Installed
	
	

	
	Data Card #1Write Verified
	
	#1 MB:

	
	Data Card #1 Installed
	
	

	
	Desiccant (new or refurbished)
	

	
	Humidity Card
	

	
	Pressure Sensor orifice screw clear
	Rating (if known):

	Bench Tests (AC Power when available)
	AC power or    Battery

	
	CB? – Baud Rate:     9600             115200    Other:

	
	PS0  Verify instrument S/N, & Freq & Firmware match with above

	
	PA    CPU;   Cards Detected as installed;  Recorded tests Pass 

	
	Failure(s):

	
	Follow up tests (as needed)      PT__  ;   PT__  ;   PT__

	
	PC2   Heading;    Pitch;    Roll;       Orientation;    Temp;    Pressure

	
	PC1                          ALL Beams Pass?
	

	
	TS (GMT Casablanca)   Computer clock verified with:

	
	Test Log file:

	
	Dummy Plug?
	

	Completed By:                                                             Affiliation:                               Date:

(Place this form with/in ADCP to be shipped to field location)

	Field Assembly & Adjustments 

	INIT
	Task
	Notes

	
	Battery connected
	 Voltage:                                                    VDC

	
	Data Cards Secured
	

	
	Desiccant secured with Rubber Band
	

	
	Humidity Card
	

	
	O-Ring inspected, lubed & installed
	 Top;    Bottom

	
	Beam-3 Alignment
	

	
	Bolts tightened to: Top 9.6 NM (7.1 ft lbs)  ; Bottom 5.6 NM (4.1 ft lbs)

	
	Compass field adjustment (AF)
	Before
	After

	
	Adjustment Log File:

	
	
	Notes:

	
	CZ
	

	
	Dummy Plug present
	

	Completed By:                                                             Affiliation:                               Date:




	Deployment Tests and Setup                         TRDI SN:____________________

	INIT
	Task
	Notes

	
	Setup Log File ([F3] on Bbtalk;  copy-paste on SC):  

	
	CB? – Baud Rate: 9600  115200    Other:

	
	OL

	
	TS (GMT Casablanca, Monrovia, Reykjavik) 
	Computer clock verified with:

	
	AX
	Total Error:

	
	PA    CPU;   Recorded tests Pass;  Cards detected as installed? □ #0;  □ #1     

	
	 DSP Test Pass  System Tests Pass (Narrow or wide may not pass if not in water) 

	
	Failures:

	
	Follow up tests (as needed)    PT200  ;   PT__  ;   PT__

	
	PC2   Heading;   Pitch;   Roll;   Orientation;  Temp;   Pressure

	
	PC1  
	ALL beams pass?

	
	AZ                                        AP? : 

	
	*.whp file uploaded  
	File location:

	
	Verify CF111n1

	
	RR - MAKE SURE DATA HAS BEEN SAVED BEFORE DELETING

	
	RN
	Name:

	
	Verify commands uploaded are as planned
	CK
	CS

	
	Apply transducer paint as needed

	Notes:



	Completed By:                                                             Affiliation:                               Date:



	Final Inspection

	INIT
	Task
	Notes
	

	BEFORE INSTALLING INTO PLATFORM

	
	Dummy plug is secured
	

	
	O-rings appear to be seated properly
	 Top;    Bottom

	
	Pressure orifice looks clear
	

	AFTER INSTALLING INTO PLATFORM

	
	Unit is secured in platform
	Type:
	Designation:

	
	Unit is pinging
	

	
	Antifoulant applied?
	Type:

	
	Yellow cover is off
	

	Notes:

	Completed By:                                                             Affiliation:                               Date:



	Final Received By (NOAA Rep):                                                                                 Date:


FILE IN STATION BINDER 

&

 Send a copy to the ED Chesapeake Instrument Lab

Please note any clock drift on the DEPLOYMENT logs after recovery.
NCOP TRDI CHECKLIST

Page 1
Version 1.2, Jan 2009

NCOP TRDI CHECKLIST

 Page 2

Version 1.2, Jan 2009


